C182Q N7569S

checklist

PRE FLIGHT CABIN

- Control Lock & Pitot Cover – Remove

- AROW Documents - Check

- Fire Extinguisher – Green

- Fuel Selector - Both

- MAGS – Off / Key Out

- Radio Master – Off

- Master Switch – On

- Flaps – Down

- Fuel Quantity – Check

- Radio Master – ON/Check Fan/OFF

  - Ext. Lights/Stall Horn/Pitot Heat – Check

  - Ext Lights/Pitot Heat – Off

  - Master Switch – Off

PRE FLIGHT EMPENNAGE

- Baggage Door – Secured and Locked

- Autopilot Static Source (2) - Clear

- Tie Down – Remove

- Surfaces & Controls – Check

- Antennas – Check

PRE FLIGHT RIGHT WING

- Surfaces & Controls – Check

- Tie Down / Chocks – Remove

- Strut / Tire / Brake – Check

- Fuel Sump – Drain

- Fuel Quantity – Check Visually

- Fuel Cap – Secure

PRE FLIGHT NOSE

- Static Source - Check

- Strut and Tire – Check

- Propeller / Spinner – Check

- Cowling/Intakes - Check

- Air Filter – Check

- Oil Filler Cap - Secure

- Engine Oil – Check (Min 9 qts.)

- Fuel Strainer - Drain

- Static Source – Check

PRE FLIGHT LEFT WING

- Pitot Tube – Check

- Fuel Vent – Check

- Stall Horn – Check

- Surfaces & Controls – Check

- Tie Down / Chocks – Remove

- Strut / Tire / Brake – Check

- Fuel Sump – Drain

- Fuel Quantity – Check Visually

- Fuel Cap – Secure

BEFORE STARTING ENGINE

- Preflight Inspection – Complete

- Passenger/Pilot Briefing – Complete

- Seats & Seatbelts – Adjust & Lock

- Cowl Flaps- Open

- Fuel Selector Valve – Both

- Alternate Static Source – Check then Off

- Circuit Breakers – In

- Electrical Equipment – Off

- Radio Master – Off

- Autopilot/Trim - Off

STARTING ENGINE

- Brakes – On

- Rotating Beacon – On

- Propeller – Visually Clear

- Master Switch – On

- Mixture – Full Rich

- Propeller – High RPM

- Throttle – Open ½ inch.

- Oil Temp – Check Hot or Cold

- Prime – 4-6 Strokes

- If HOT – 1-2 Strokes

- Primer - Locked

- Propeller – Verbally “CLEAR!”

  - MAGS – Start

  - Throttle – 1000 RPM

  - Oil Pressure – Check

  - Ammeter – POS. Charge (+)

  - Mixture – Lean 1”

PRE TAXI

- Radio Master– On

- NAV Lights – On (As Req.)

- Flaps – Retract

- ATIS / AWOS – Obtained

- Altimeter- Set

- Flight Instruments – Set and Check

- Fuel Totalizer – Set (As Req.)

- GPS Database – Check

- Garmin 696- Check

- Taxi Light – As Required

- Clearance – Obtained

- Transponder – Code Set

TAXI

- Brakes – Test (L/R/BOTH)

- Flight Controls – Wind Correction Set

- Instrument Check – IFR / VFR Check

- Mixture – Rich (As Required)

ENGINE RUNUP

- Brakes – On

- Mixture - Rich

- Flight Controls – Free & Correct

- Elevator/ Rudder Trim – Set for Takeoff

- Throttle – 1700 RPM

- MAGS (R&L ; 150/50)

- Propeller- Cycle RPM (3 Times)

- Carb Heat - Check

- VAC Ind. – Green

- Engine Ind. – Check

- Ammeter – Check POS Charge

- Idle RPM – Check (575-675 RPM)

- Throttle – 1000RPM

- Throttle Friction – Set

- COM / NAV – Set

- Flaps – Set (0°-20°)

- Parking Brake – Check Off

BEFORE TAKEOFF

- Windows and Doors – Closed & Latched

- Fuel Selector Valve – Both

- Engine Instruments – Check

- Carb Heat - Off

  - Transponder – Code Set

  - Landing Light and Strobe Lights – On

  - Cowl Flaps - Open

  - Departure Briefing - Complete

════════════════

  - Runway Heading – H.I. /Compass Check

  - Takeoff Time – Note

AFTER TAKEOFF

- Airspeed – 85-95 KIAS

- Power – 23” & 2400 RPM

- Mixture – Rich (Lean above 5,000 ft)

Flaps – Retract X WIND MAX DEMO – 15

Vr – 50-60

Vx – 54

Vy – 78

Vso – 45

Vs – 48

Best Glide – 70

Va (1950 lbs) – 89 (2450 lbs) - 100 (2950lbs) – 111

Vno – 143

Vne – 179

Vfe (10°) -140 / (20° - Full) 95

- Engine Instruments – Check

- Landing Light – Off

CRUISE

  - Power – 15”-23” 2000-2400 RPM

  - Mixture – Lean

  - Engine Instruments – Check

  - Cowl Flaps - Closed

DESCENT / APPROACH

- ATIS / AWOS – Obtained

- Altimeter – Set

- Pre-Landing Brief – Complete

- Com / Nav – Set

- Fuel Selector Valve – Both

- Carb Heat – As Required

- Mixture – Full Rich

- Landing Light – On

BEFORE LANDING

- Flaps – As Required

- Propeller – High RPM

- Mixture – Rich

- Carb Heat - On

- Autopilot/Trim - Off

AFTER LANDING

- Flaps – Retract

- Cowl Flaps – Open

- Carb Heat - Off

- Pitot Heat – Off

- Landing/Taxi Light – As Req.

- Strobe Lights - OFF

- Mixture – Lean 1”

SECURING

- Radio Master– Off

- Throttle – 1000 RPM

- Electrical Switches – Off (BCN ON)

- Mixture – Idle Cut Off

- MAGS – Off / Key Out

- Master Switch – Off

- Fuel Selector – Left or Right

- Tach  / MX Squawks – Record

- Control Lock /Tie Downs / Pitot Cover

- Flight Plan – CLOSED!

KCMI Frequencies

ATIS – 124.85

Clearance – 128.75

Ground – 121.8

Tower – 120.4

Approach/Departure – 121.35 N/E

Approach/Departure - 132.85 S/W

CMI VOR – 110.00