C182Q N7569S

Emergency checklist

Procedures shown in bold faced type are immediate action items which should be committed to memory.

For amplified emergency procedures refer to POH Sec 3.

Transponder Codes:

7700 – Emergency

7600 – Lost Communications

7500 – Hijack

Emergency Frequency – 121.5

ENGINE FAILURES

DURING FLIGHT (RESTART)

IF TIME PERMITS

- Airspeed – 70 KIAS Flaps Up

- Landing Site – SELECT

- Carb Heat - ON

- Fuel Selector Valve – BOTH

- Mixture – RICH

- Throttle – SET

- Primer - LOCKED

- MAGS – Both or Start

FORCED LANDING

IF ENGINE FAILS TO RESTART

- Airspeed – 70 KIAS (Flaps Up)

- Fuel Selector Valve – OFF

- Mixture – IDLE CUT OFF

- MAGS – OFF

- Cabin – SECURE

- Declare emergency

- Flaps – AS REQUIRED

- Master Switch – OFF

- Cabin Doors – UNLATCH

- Touchdown – TAIL LOW

- Brakes – APPLY HEAVILy

FIRES

DURING START ON GROUND

- MAGS – START

Continue cranking

IF ENGINE STARTS

- Power – 1700 RPM 2-3 MIN

- Engine – SHUTDOWN /

INSPECT FOR DAMAGE

IF ENGINE FAILS TO START

- Throttle – FULL OPEN

- Mixture – Idle Cut Off

- MAGS – START

Continue cranking

- Fuel Selector Valve – OFF

- Engine – SECURE

A. Master Switch - Off

B.  MAGS – OFF

- Parking Brake – RELEASE

- Fire Extinguisher – OBTAIN

- Airplane – EVACUATE

- Fire – EXTINGUISH

- Fire Damage – INSPECT

ELECTRICAL FIRE IN FLIGHT

- Master Switch – Off

- Vents/Cabin Air/Heat – Closed

- Fire Extinguisher – ACTIVATE

- Radio Master – OFF

- Electrical Switches – OFF

VERIFY FIRE IS EXTINGUISHED

- Vents/Cabin Air/Heat – OPEN

- Master Switch – ON (AS REQ.)

- Circuit Breakers – CHECK

DO NOT RESET

- Radio Master – ON (AS REQ.)

- Land – At Nearest Suitable Airport

CABIN FIRE

- Master Switch – Off

- Vents/Cabin Air/Heat – Closed

- Fire Extinguisher – ACTIVATE

VERIFY FIRE IS EXTINGUISHED

- Vents/Cabin Air/Heat – OPEN

- Land – At Nearest Suitable Airport

WING FIRE

- NAV and Strobe Light Switch – OFF

- PITOT HEAT Switch – OFF

- Sideslip flames away from fuel tank and cabin.

VERIFY FIRE IS EXTINGUISHED

- lAND – ASAP

- Flaps – ONLY AS REQ ON FINAL APPROACH

ENGINE FIRE IN FLIGHT

- Mixture – IDLE CUT OFF

- Fuel Selector Valve – OF

- Master Switch – OFF

- Cabin Heat & Air – OFF

OVERHEAD VENTS OPEN

- Airspeed - ≥ 100 KIAS

- Forced Landing – EXECUTE

ROUGH RUNNING ENGINE OR PARTIAL LOSS OF POWER

SPARK PLUG FOULING

SLIGHT ENGINE ROUGHNESS

- Carb Heat - On

- MAGS – CHECK (R&L)

- Mixture – LEAN FOR CRUISE        (SEE POH)

- Mixture – Enrich if problem persists

- Land – At Nearest Suitable Airport

MAGNETO MALFUNCTION

SUDDEN ENGINE ROUGHNESS

- MAGS – CHECK (R&L)

- Mixture – FULL RICH

- Throttle – SET (AS REQ)

- If Problem Persists switch to good MAG

- Land – At Nearest Suitable Airport

LOW OIL PRESSURE

OIL PRESS ANNUNCIATOR

- OIL PRES Indicator – CHECK

- OIL TEMP Indicator – CHECK

IF BOTH INDICATING NORMAL

- Land – At Nearest Suitable Airport

IF INDICATING LOSS OF OIL PRES

AND RISE IN OIL TEMP

- Power – REDUCE IMMEDIATELY

- Forced Landing – EXECUTE